Dashboard folder structure:

- INF1006_Group16_Project\smart_home\backend

- NF1006_Group16_Project\smart_home

- INF1006_Group16_Project\camera\Camera

Telegram Bot folder structure:

- INF1006_Group16_Project\telegram

Sentinel Pi Folder structure:

- Sensor_con

Eco_guard Pi Folder structure:

- Actuator_con

Dashboard startup guide:

- Terminal 1(Backend):

  - cd INF1006_Group16__Project\smart_home\backend

  - Run: uvicorn server:app --host 0.0.0.0 --port 8000

- Terminal 2(Web-Camera):

  - cd INF1006_Group16__Project\camera\Camera

  - Run: uvicorn server:app --host 0.0.0.0 --port 5001

- Terminal 3(Frontend):

  - cd INF1006_Group16__Project\smart_home

  - Run: npm run dev

Common error debugging:

- If the backend or camera folder uvicorn is not recognised, in either terminal run

  - pip install uvicorn

- Frontend missing dependencies, run:

  - npm install

  - Re-run: npm run dev

Telegram Bot(HOMIE) startup guide:

- Terminal 1:

  - cd INF1006_Group16_Project\telegram

  - python main.py

Common error debugging:

- If python main.py does not work run:

  - python3 mian.py

Sentinel Pi startup guide:

- Copy the folder sensor_con into the raspberry pi

- In the config.py change the dashboard ip to the ip address of the device running the dashboard

- In terminal 1:

  - cd sensor_con

  - python3 main.py

Pin setup:

- Power: RPi 3.3V (Physical Pin 1) connects to MCP3008 VDD (Pin 16) and VREF (Pin 15).

- Ground: RPi GND (Physical Pin 9) connects to MCP3008 DGND (Pin 9) and AGND (Pin 14).

- Serial Clock: RPi SPI0 SCLK (Physical Pin 23) connects to MCP3008 CLK (Pin 13).

- Master Out Slave In: RPi SPI0 MOSI (Physical Pin 19) connects to MCP3008 DIN (Pin 11).

- Master In Slave Out: RPi SPI0 MISO (Physical Pin 21) connects to MCP3008 DOUT (Pin 12).

- Chip Select: RPi SPI0 CE0 (Physical Pin 24) connects to MCP3008 CS (Pin 10).

- Pir OUT Pin: GPIO24 (Physical Pin 18)

- Pir VCC: 5V

- DHT11 DATA/OUT Pin: GPIO4 (Physical Pin 7)

- DHT VCC: 3.3V

Common error debugging:

For module error:

  - No module named 'spidev'

  - No module named 'board'

  - No module named 'adafruit_dht'

  - No module named 'gpiozero'

  - No module named 'requests'

- In the pi terminal run:

  - sudo apt update

  - sudo apt install python3-pip -y

  - pip3 install requests gpiozero adafruit-circuitpython-dht adafruit-blinka spidev

- If permission error surface:

  - python3 -m pip install --break-system-packages requests gpiozero adafruit-circuitpython-dht adafruit-blinka spidev

- Re-run:

  - python3 main.py

For SPI rain sensor error:

- Enable SPI on the raspberry pi:

- Terminal run: sudo raspi-config

- Then go to:

  - Interface Options

  - SPI

  - Enable

- Terminal run: sudo reboot

Eco-guard Pi startup guide:

- Copy the folder actuator_con into the raspberry pi

- Terminal 1:

  - cd actuator_con

  - python3 app.py

Pin layout:

- Window Servo 1: GPIO17(Physical Pin 11)

- Window Servo 2: GPIO27(Physical Pin 13)

- Door Servo: GPIO22(Physical Pin 15)

- Fan Servo:  GPIO23(Physical Pin 11)

- LED 1: GPIO5 (Physical Pin 29)

- LED 2: GPIO6 (Physical Pin 31)

- LED3: GPIO13(Physical Pin 33)

Common error debugging:

ModuleNotFoundError for Flask, pigpio, or gpiozero:

- Terminal 1:

  - sudo apt update

  - sudo apt install python3-pip -y

  - pip3 install flask gpiozero pigpio

- If pip3 install flask gpiozero pigpio fail run:

  - python3 -m pip install --break-system-packages flask gpiozero pigpio

- Re-run:

  - python3 app.py

RuntimeError: Could not connect to pigpio daemon. Run: sudo systemctl start pigpiod

- Terminal 1:

  - sudo systemctl start pigpiod

  - sudo systemctl enable pigpiod

Citation:

Anthropic. (n.d.). Claude. https://claude.com/product/overview

Canva. (n.d.). AI voice generator: Text to speech online. https://www.canva.com/features/ai-voice-generator/

OpenAI. (n.d.). ChatGPT. https://openai.com/chatgpt/
Raspberry Pi. (n.d.). Raspberry Pi. https://www.raspberrypi.com/

AI Usage:

Canva AI voice was used to generate narration for the video script created by us (Canva, n.d.). ChatGPT and Claude were used for brainstorming, wording support, error checking and idea refinement (Anthropic, n.d.; OpenAI, n.d.).”